2.23

Driving and reversing

2.23.1 The fully laden solar car must be able to start from rest on a 7% gradient.

2.23.2 The solar car must be able to be driven backwards under its own power with the driver seated

in the normal position.

2.24

Lighting

2.24.1 Solar cars must be fitted with:

• two rear stop lamps

• one central stop lamp

• left and right front direction indicator lamps

• left and right-side direction indicator lamps

• left and right rear direction indicator lamps35

• two daytime running lamps.

Headlamps and tail lamps are not required, though would increase the practicality score of Cruiser Class solar cars.

2.24.2 Stop lamps must emit red light. Direction indicator lamps must emit amber light. Daytime

running lamps must emit white light.

2.24.3 Lamps must be compliant with UNECE Regulations 6, 7 and 87, or the SAE/DOT equivalents.

Teams must demonstrate compliance by either:

• the presence of compliance markings on the lamps, or

• detailed documentation that demonstrates compliance with the photometric requirements

of the UNECE or SAE/DOT regulations, confirmed by the team’s certifying engineer.

Lamps approved for motorcycles may not meet these requirements.

2.24.4 Solar cars must have the correct type of lamp in each position. Lamps must be mounted with

the correct orientation so that the photometric requirements of UNECE Regulations 6, 7 and 87,

or the SAE/DOT equivalents, are met.

Lamp

UNECE category

SAE/DOT type

Front indicators

1, 1a, 1b

I3, I4, I5

Rear indicators

2a, 2b

I6

Side indicators

5, 6

E2

Stop lamps

S1, S2

S

Central stop lamp

S3

U3

Daytime running lamp

RL

Y2

Lamp position and visibility requirements are based on UNECE Regulation 48.

2.24.5 Rear stop lamps must be within 400 mm of the extreme outer edge of the solar car on each side,

at least 600 mm apart (at least 400 mm apart if the solar car is less than 1300 mm wide), and at

least 350 mm above the ground. The entire apparent surface must be visible 15° up, 5° down

and 45° to the left and right.

The “apparent surface” of a lamp includes all parts of the lamp surface that emit light and are not obscured by other

parts of the light-emitting surface.

2.24.6 A central stop lamp is required. Viewed from behind the solar car, the lateral position of the

lamp must coincide with the visual centre of the solar car (see the examples in the following

diagram). The lamp must be higher than a point 150 mm below the rear windscreen (if the solar

car has a rear windscreen), and the bottom of the lamp must be higher than the top of the rear

stop lamps. The entire apparent surface must be visible 10° up, 5° down and 10° to the left and

right.2.24.7 Front and rear direction indicator lamps must be within 400 mm of the extreme outer edge of

the solar car on each side, at least 600 mm apart (at least 400 mm apart if the solar car is less

than 1300 mm wide), and at least 350 mm above the ground.

2.24.8 Side direction indicator lamps must be less than 1800 mm behind the front most part of the

solar car and within 400 mm of the extreme outer edge of the solar car on each side.

2.24.9 The entire apparent surface of direction indicator lamps must be visible 15° up and 5° down.

Minimum horizontal visibility requirements (of the right direction indicator lamps) are shown in

the following diagram.

2.24.10 Direction indicators must flash at 90±30 flashes per minute.

2.24.11 It must be possible to flash the left and right direction indicator lamps simultaneously, as a

hazard warning signal.

2.24.12 Daytime running lamps must be mounted at the front of the solar car, at least 600 mm apart (at

least 400 mm apart if the solar car is less than 1300 mm wide), and at least 250 mm above the

ground. The entire apparent surface must be visible 10° upwards, 10° downwards, 20° outwards

and 20° inwards.

2.24.13 Headlamps (if fitted) must be at the front of the solar car within 400 mm of the extreme outer

edge of the solar car on each side, at least 600 mm apart (at least 400 mm apart if the solar car

is less than 1300 mm wide), and at least 500 mm above the ground. The entire apparent surface

must be visible 15° upwards, 10° downwards, 45° outwards and 10° inwards.

2.24.14 The stop lamps must operate whenever driving is possible and the brakes are applied.

3637

2.24.15 The daytime running lamps must operate whenever driving is possible.

2.25

Audible warning device

2.25.1 An audible warning device complying with the intent of UNECE Regulation 28 must be fitted to

the solar car.

2.25.2 Sound pressure level must be more than LA = 105 dB measured 2 m from the horn.

The horn should be mounted so that solar car occupants are not subjected to excessive sound pressure levels.

2.25.3 The device must emit a continuous and uniform sound. The audible warning device must be

capable of operating for 50,000 cycles of on for one second and off for four seconds.

2.26

Instrumentation

2.26.1 The following information must always be visible to the driver (i.e., not behind menus) while the

car is in motion:

• the speed of the solar car

• whether the direction indicators are operating

• whether the hazard lights are operating

• energy storage system warnings

• electronic rear vision images (if fitted).

2.26.2 This instrumentation must be powered from the energy storage system, and not from separate

batteries.

2.27

Automatic functions

2.27.1 Any cruise control function must automatically deactivate when the brake is operated, or the

car is turned off.

Cruise control must not resume automatically following a brake operation.

2.27.2 Any automatic driving function must immediately deactivate on manual input or when the car

is turned off.

Automatic driving must not resume automatically.

2.28

Electrical safety

Electrical safety requirements are based on Section 5 of UNECE Regulation 100. The term ‘high voltage’ means more

than 60 V dc or more than 30 V rms ac.2.28.1 Protection against direct contact with high-voltage parts, including conductors, must be

achieved using double insulation, enclosures or barriers. It must not be possible to remove

protection without the use of tools.

2.28.2 Protection against direct contact with high-voltage parts inside the driver, passenger and

luggage compartments must be designed to exclude objects larger than 1 mm diameter (Ingress

Protection rating IPXXD).

2.28.3 Protection against direct contact with high-voltage parts inside areas other than the driver,

passenger and luggage compartments must be designed to exclude fingers (Ingress Protection

rating IPXXB).

2.28.4 Double insulation must meet the AS 3001/IEEE 100 definition: comprising both basic insulation

and independent supplementary insulation that provides protection equivalent to that of the

basic insulation. A single layer of reinforced insulation is also acceptable if it provides protection

equivalent to double insulation.

Electrical tape and flimsy, easily displaced covers are unlikely to meet the requirements of supplementary insulation.

The front surface of a photovoltaic cell is deemed to be double-insulated if properly encapsulated.

2.28.5 High-voltage energy storage packs must be marked with the high-voltage symbol shown in the

following diagram.

2.28.6 The high-voltage symbol must also be visible on any enclosure or barrier that can be accessed

without using tools, if removing the enclosure or barrier exposes high-voltage parts.

Example: A motor controller contains high voltage parts, so these parts must be protected by an enclosure or barrier

that requires tools to remove (Regulation 2.28.1). If it is possible to access the motor controller enclosure without

using tools (e.g. by opening the boot or tilting the solar collector) then the motor controller enclosure must have a

high-voltage symbol on it.

2.28.7 The resistance between any exposed conductive part and each terminal of the energy storage

system must exceed 100 V ohms, where V is the nominal voltage of the energy storage system.

This is equivalent to a maximum leakage current of 10 mA.

2.28.8 The resistance between any exposed conductive part and each terminal of every solar cell must

exceed 100 V ohms, where V is the maximum circuit voltage of the solar collector.

2.28.9 A system must be implemented to monitor the isolation of the solar car’s chassis and body as

per Regulations 2.28.7 and 2.28.8. This system may take the form of a permanently connected

electronic device or a properly documented procedure of regular manual checks.

2.28.10 A fuse or circuit breaker, suitably rated to protect the wiring, must be mounted in or on each

energy storage pack. Additionally, the dc interrupting current (i.e., dc breaking current) capacity

3839

of the device must be able to interrupt the maximum possible short-circuit current of the battery

packs.

A typical solar car might use a 50 A fuse with a 15000 A dc interrupt rating. Fuses and circuit breakers intended for ac

use might not meet this requirement. Teams must provide a manufacturer’s data sheet clearly showing that the dc

specifications of the device are adequate.

2.29

Electrical safe state

2.29.1 The solar car must have a ‘safe state’ which, in an emergency, minimises the risk of electrical

fire and electric shock to occupants, team members, emergency response personnel, and

bystanders.

Safe state is for emergencies and for complete shutdown of the car. In addition to safe state, a solar car may have a

“standby” state that provides power to some subsystems outside of the energy storage packs.

An external battery is not necessary to bring the car out of safe state. Possible alternatives include:

• a switch on the energy storage pack

• an air switch inside an energy storage pack, with an airline to a remote start button

• a fibre-optic switch.

When in the safe state:

• every conductor emerging from each energy storage pack must be galvanically isolated from

every energy storage cell

• no voltage may be present across any pair of conductors emerging from energy storage

packs or the solar collector

• no current may be present through any conductor loop that is external to the energy storage

packs or the solar collector.

MOSFETS and other semiconductor devices are not considered to offer galvanic isolation.

2.29.2 Any conductor that is more than 200 mm from the nearest PV cell is outside of the solar

collector.

2.29.3 All mechanisms for placing the solar car into safe state and maintaining safe state must be fail

safe; if an electrical activation mechanism fails, the solar car must automatically and

immediately place itself into safe state and must remain in safe state indefinitely.

A simple design might use normally-open contactors in the energy storage packs, and have these contactors energised

via a series loop of mechanical switches, all of which must be closed for normal solar car operation. If the loop breaks

or any switch opens, the contactors will open, power to the loop will be removed, and the vehicle will enter safe state.

Teams using other mechanisms, such as those incorporating solid state switches (which can fail closed) or software

(which can fail in many ways), must be able to demonstrate beyond reasonable doubt that the probability of the

mechanism failing to place the car into safe state is less than that of a system with normally-open contactors and

normally-closed switches.40

2.29.4 The driver must be able to place the solar car into safe state with a single simple action while

seated in the normal driving position and without releasing the safety-belt. This activation device

must be clearly labelled.

2.29.5 For emergency use, an activation device that operates with a single simple action that

immediately places the solar car into safe state must be provided on the exterior of the car.

Multiple activations must not bring the car out of safe state. The activation device must be

placed within a yellow disc with a diameter of 180 mm. Also, in the yellow disk must be a blue

equilateral triangle (minimum side length 150 mm) that contains a red flash, with the legend

Emergency Electrical Isolation. In addition, there must be a clear instruction on how to operate

the device (e.g., PULL or PRESS). The yellow isolation disc and the activation mechanism must

be clearly visible to an emergency services first responder approaching the driver and must be

within 100 mm of the base of the windscreen, adjacent to the driver egress opening, and not

behind the driver.

Both activation mechanisms must be demonstrated at static scrutineering. The scrutineers may invite random

members of the public to validate the ease of operation of the external emergency activation device.41

3

The Adventure

3.1

Supervision

3.1.1

The Clerk of the Course is responsible for supervising on-road activities.

3.1.2

During on-road activities, communication with the Clerk of the Course must be made through

Mission Control.

3.2

Safety

3.2.1

The team safety officer is responsible for the general safety of the team.

3.2.2

The team battery officer is responsible for the safe operation of batteries and for supervising

response to any battery emergency.

3.2.3

Battery incident response kits must be readily available to the solar car and any other vehicle

carrying solar car batteries.

3.2.4

A team member with a recognised first-aid certificate, current for the duration of the event,

must travel in the front or rear escort vehicle.

3.2.5

It is the responsibility of each team to ensure that the vehicles under its control are maintained

in a safe, roadworthy condition and are operated safely and within the law always.

3.2.6

All solar cars and escort vehicles are operated and driven at the team’s own risk.

3.2.7

The Clerk of the Course may exclude any team from the event at any time if any team member

operates a solar car or any other vehicle in an unsafe manner.

3.2.8

Teams must look after the health and safety of their team members and observer.

Fatigue and dehydration are serious hazards in this event. This is especially relevant to drivers, particularly those of

solar cars.

3.2.9

Drivers, team members, officials and observers are to be drug free and maintain a blood alcohol

level less than 0.01% whilst engaged in any duties associated with the event.

3.3

Safety-belts

3.3.1

The use of safety-belts is mandatory for all occupants of motor vehicles in Australia, including

solar cars.

3.4

Helmets

3.4.1

Each occupant of a solar car must wear an approved motorcycle helmet securely fitted and

fastened in accordance with the manufacturer's instructions.42

3.4.2

Motorcycle helmets must have a mark certifying compliance with either:

• Australian/New Zealand Standard AS/NZS 1698:2006

• United Nations Economic Commission for Europe Regulation No 22 (UNECE 22.05).

Half-shell helmets are not permitted.

3.4.3

Helmets must not be modified, have unauthorised attachments, or be used in any way contrary

to the manufacturer’s instructions.

3.5

Safety equipment

3.5.1

Suitable and appropriate safety equipment must be carried always. The minimum safety

equipment that must be carried in the rear escort vehicle is:

• a first-aid kit

• safety glasses and gloves for handling batteries

• at least two hazard warning cones

• two yellow warning flags, minimum size 300 mm x 300 mm

• suitable fire extinguishers

• fire blankets

• sand (or similar material) for extinguishing fires

• a spade

• suitable containers for damaged electrochemical cells

• safety vests

• two whistles.

3.6

Support vehicles

3.6.1

Each team must supply a front and a rear escort vehicle, a means of transporting the solar car,

and sufficient additional vehicles to transport and support the team and its equipment during

the event.

3.6.2

The minimum requirement for transporting the solar car is a suitable truck or trailer, which must

accompany the team throughout the journey.

Many vehicle rental companies in Australia require the hirer and all drivers to be at least 23 years of age. One-way

interstate rentals may not be possible through an airport rental desk and require head office approval. Trailers should

be booked early.

3.6.3

Each solar car must, always when in motion on the route, be accompanied by two escort

vehicles—one immediately in front of the solar car and one immediately behind the solar car.43

3.6.4

A minimum of two drivers must be available for every vehicle associated with the team.

3.6.5

The rear escort vehicle must not be a bus, truck or large campervan, or be towing a trailer.

If traffic approaching from behind sees a large vehicle with a flashing light, the immediate perception is that that

vehicle itself is the hazard, not the solar car it is escorting. This may lead to a member of the public overtaking a large

rear escort vehicle only to encounter an unexpected and previously unseen solar car. Large rear escort vehicles make

it difficult for vehicles approaching from the rear to see the solar car.

3.7

Escort vehicle signage

3.7.1

All team vehicles must carry a sign, visible from 30 m to the rear, stating the name and team

number of the team to which they belong.

3.7.2

All team vehicles equipped with Citizen’s Band (CB) radio must carry a sign, visible from 30 m to

the rear, advising the CB channel number in use.

3.7.3

A warning sign larger than 900 mm x 300 mm with black lettering on a yellow background, clearly

visible from 30 m, must be displayed on the rear of the rear escort vehicle, stating

CAUTION: SOLAR VEHICLE AHEAD

3.7.4

No vehicle other than the rear escort vehicle may carry such a sign.

3.7.5

Front and rear escort vehicles must be fitted with amber flashing lights visible from all directions

at a distance of at least 200 m in daylight. Flashing amber lights must not be obscured by signs

or other equipment on the roof of an escort vehicle.

Beacons designed as warning devices for industrial equipment are unlikely to meet the visibility requirement.

3.8

Communication

3.8.1

Every solar car must have means of two-way voice radio communication with its rear escort

vehicle. Communication must be possible whenever the driver is in the solar car, including when

the main energy storage contactors are open and when the car is in safe state.

3.8.2

Each team must carry a satellite telephone and notify the event organiser of its number. This

notification will be required at registration. An external antenna is usually required when using

a satellite telephone in a vehicle.

3.8.3

The front and rear escort vehicles (at least) for each solar car must each have an 80-channel UHF

CB radio compliant with Australian regulations. Teams must monitor the channel they have been

allocated always while on the road, and respond promptly and courteously, in English, to

reasonable communications with other road users, teams and officials.

3.9

Heavy vehicles

3.9.1

Any team operating or contracting a vehicle over 4.5 tonnes GVM or a passenger vehicle with

12 or more seats must register the full details with the event organiser.44

3.9.2

Compliance with Australian National Heavy Vehicle regulations concerning driving hours, record

keeping and fatigue management is mandatory.

3.10

Preparation and testing

3.10.1 The event organiser will provide facilities for team preparation and track testing of solar cars at

the Hidden Valley Motor Sports Complex in accordance with the dates listed in the schedule.

3.10.2 All participating teams must base themselves, and their solar cars, at the Hidden Valley Motor

Sports Complex from Monday 9 October 2023.

3.10.3 The event organiser will appoint a paddock manager to supervise the preparation and test

facility. The directions of the paddock manager must be followed.

3.10.4 Australian workplace standards will apply to all event-related activities in Australia.

3.10.5 The Clerk of the Course may impose event penalties for breaches of speed limits or other rules

at the Hidden Valley Motor Sports Complex.

3.10.6 No test driving is to be conducted in the paddock, car park or access roads.

3.10.7 The track must not be accessed without clearance from the track controller.

3.10.8 The event organiser will work with the relevant authorities to provide on-road testing facilities.

Detailed arrangements will be published in the Team Manager’s Guide and/ or via a Team

Notice.

3.10.9 Any team driving a solar car on a public road without a permit is liable to civil penalties and

exclusion from the event.

Teams who make direct contact with Australian road authorities delay this process.

3.10.10 The event organiser reserves the right to request a security deposit against damage, rubbish

removal or cleaning beyond reasonable expectations.

3.11

Team registration and scrutineering

3.11.1 All participants are required to attend with their team to complete the registration process and

present their solar car for the scrutineering process. The scrutineering process is divided into

static and dynamic operations.

3.11.2 Team registration and static scrutineering will take place at the Darwin Convention Centre or

such other place as determined by the event organiser at the time and date described in the

schedule.

3.11.3 Teams must attend registration and static scrutineering with:

• the Team Manager, and an interpreter if required45

• all solar car drivers, with their driving licences

• any waiver documents required for participants under the age of 18

• original signed copy of the indemnity form

• all solar car passengers

• the team safety officer

• the team battery officer

• the solar car, in road-ready condition

• front and rear escort vehicles, including any trailer that may be attached to the front escort

vehicle, in road-ready condition

• the solar car trailer or solar car transport truck they intend to use in the event

• tools and personnel required to facilitate inspection of the solar car

• a list of all vehicles associated with the team, including make, model, colour and registration

number

• Group D compulsory documentation

• a spare solar car tyre of each type

• a sample energy storage cell (if cells inside the energy storage packs are not clearly visible)

• the lockable box in which energy storage packs can be impounded where required.

3.11.4 Solar cars must be transported to scrutineering on the solar car trailer or truck that will be used

during the event.

3.11.5 The Team Manager must attend all registration and scrutineering stations. A Team Manager not

proficient in English must provide an interpreter to accompany them always during this process.

3.11.6 All team members must complete their online registration by signing in at registration.

3.12

Static scrutineering

3.12.1 Static scrutineering will check for compliance with the regulations and will include a regulatory

compliance and roadworthiness inspection by the Northern Territory road transport authority.

3.12.2 Qualification must be achieved in road-ready condition. Teams that do not present at the

designated time in road-ready condition may fail to qualify.

3.12.3 Up to seven members of a team (including an interpreter and any team media personnel) may

accompany the solar car on the scrutineering floor. Team members may be substituted between

inspection stations.46

3.12.4 Some dismantling (such as removal of the solar collector or top shell) may be required to

facilitate inspection of the following components:

• mechanical systems (including seats, tyres, brakes and steering)

• electrical systems

• energy storage system.

3.12.5 Dismantling and reassembly of the solar car when required to facilitate inspection of the solar

car must be conducted (in the sole opinion of the Chief Scrutineer) in a reasonable time.

3.12.6 Checks and inspections with the solar car in a road-ready configuration will include, but not be

limited to:

• signage

• dimensions

• solar collector type and size

• egress—all solar car drivers and passengers are required

• vision—all solar car drivers are required

• lights, indicators and horn

• electrical compliance.

3.12.7 All energy storage devices must be declared at static scrutineering. Failure to declare any energy

storage device may lead to disqualification.

3.12.8 The mass of each Challenger or Cruiser Class solar car driver or passenger, with helmet, driving

clothes and shoes, will be determined at scrutineering. If the mass of a driver or passenger is

less than 80 kg, ballast provided by the event organiser will be added to make up the difference.

No credit will be given if a driver or passenger weighs more than 80 kg.

Deliberate consumption of excessive food and drink prior to weigh-in is not in keeping with the spirit of the event and

may be hazardous to the health of the individual.

Solar car occupants must wear similar clothes (including shoes) while driving as when they weigh in.

3.12.9 Drivers and passengers may be re-weighed at any time during the event. Changes in weight

considered unreasonable by the Chief Medical Officer will be referred to the Clerk of the Course.

3.12.10 Non-compliance penalties may be imposed at the absolute discretion of the Chief Scrutineer

and may include failure to qualify. Penalties will be published.

3.13

Dynamic scrutineering

3.13.1 Dynamic scrutineering exercises will test the speed, stability and braking capability of the solar

car.47

3.13.2 Dynamic scrutineering will be held at the Hidden Valley Motor Sports Complex, or such other

place as determined by the event organiser, on Thursday 19 October 2023 Attendance of Team

Managers and drivers at 07:00 roll call is mandatory.

3.13.3 No solar car may be submitted for dynamic scrutineering without having first passed static

scrutineering.

3.13.4 Solar cars must complete dynamic scrutineering in road-ready condition and with occupants and

ballast in place.

3.13.5 Cruiser Class solar cars with four or fewer seats must complete dynamic scrutineering with all

seats occupied. Cruiser Class solar cars with more than four seats must complete dynamic

scrutineering with four seats occupied.

3.13.6 No solar car will be given permission to start the event until it has passed dynamic scrutineering.

Any solar car failing dynamic scrutineering will only be permitted to re-present at the absolute

discretion of the Chief Scrutineer. Dynamic scrutineering will close at 12:30 on Thursday 19

October 2023.

3.14

Modifications or changes after scrutineering

3.14.1 Once a solar car and its drivers have passed static scrutineering, no changes to the design or

configuration of the solar car (that is, to any items described in the compulsory documentation)

or changes to drivers will be permitted.

3.14.2 Once a solar car has passed dynamic scrutineering, and until released from the start line,

component exchange, modification or repair of the solar car is not permitted without the

approval of the Chief Scrutineer.

3.15

Briefings

3.15.1 A compulsory briefing for Team Managers and team safety officers will be held in Darwin on

Monday 9 October 2023 at a time and place to be advised. Team managers not proficient in

English must request permission to be accompanied by an interpreter.

3.15.2 A compulsory safety, procedural and media briefing will be held on Thursday 19 October 2023

at a time and place to be advised. All team members must attend.

3.16

Observers

3.16.1 The Clerk of the Course will appoint an observer to travel with each team. Observers may be

changed between teams at any time during the event.

3.16.2 The observer's responsibilities are:48

• to record the locations and times that teams start and stop

• to record the locations and times of all activities and incidents that may affect the outcome

of the event

• to record the number of Cruiser Class occupants travelling between control stops

• to check that ballast is correct for each driver or passenger change

• to be satisfied that the energy storage system cannot be charged or interfered with between

sunset and sunrise, except as allowed for Cruiser Class solar cars

• to notify a Red-Shirt or the Clerk of the Course at the earliest opportunity if any breach of

regulation is suspected, energy storage cells or modules have been changed or an energy

storage system seal broken, or if they have any concerns about the actions of the team.

3.16.3 Observers are not permitted to interpret regulations or to give advice or directions to teams.

Advice or directions from observers do not supersede regulations or directions from Red-Shirts.

The only exception is if an observer asks a team to stop because they feel unsafe—the team

must stop.

3.16.4 Each team’s responsibilities to the observer are:

• to assist the observer in their duties always

• to provide reasonable food for the observer at least three times a day, and such drinking

water as the observer may require

• if requested by the observer, to send for assistance from the event organiser or transmit

information to Mission Control

• to provide a proper seat in the rear escort vehicle

• to provide space for the observer's luggage in the rear escort vehicle

• to not make any demands of the observer to assist team operations.

3.16.5 Each team must allocate the front passenger seat of the rear escort vehicle to the observer. This

includes fair and reasonable space with, for example, the foot well not being filled with

computer equipment such that the passenger space is restricted.

3.17

Start line and grid

3.17.1 The ceremonial start of the event will be held in State Square, Darwin, from 09:45 on Friday 20

October 2023, or such other place and time as the event organiser may direct. Detailed

procedural instructions will be issued in Darwin by Team Notice.

3.17.2 Starting position will be determined by class and performance during dynamic scrutineering.

3.17.3 All solar cars must be in their starting grid position by the appointed time for final inspections.

Any solar car not present will have their starting position amended.49

3.17.4 Teams must not work on their solar cars on the start grid.

3.17.5 Two team members must stay with the solar car on the start grid to assist officials with final

inspections.

3.17.6 Front and rear escort vehicles must be in their designated places by 09:00.

3.17.7 Escort vehicles must be in position with the solar car before entering public roads. Solar cars

which have left the start line and whose escort vehicles are not able or not ready to merge will

be directed to return to the start area.

Team managers should recognise that it is not a realistic expectation for escort vehicle crew members to witness the

start and re-join their vehicles to make a timely departure.

3.18

Energy collection and storage

3.18.1 Charging the energy storage system or powering the solar car from an unauthorised source will

lead to exclusion from the event.

3.18.2 External devices intended to increase the irradiance on the solar collector or increase the

efficiency of the solar collector must not be used at any time.

Spraying of water from external sources is not allowed.

Ground sheets must not be placed in any position where it is possible to draw a straight line from any part of the

ground sheet to any part of the solar collector.

3.18.3 From the official start of the on-road component of the event until finish or withdrawal, the

energy storage system must never be removed from the observer’s control.

3.18.4 Damaged energy storage cells may be bypassed, but must not be removed from the energy

storage system unless it would be unsafe to leave them in. Cells removed must be replaced by

ballast with mass greater than the mass of the removed cells.

3.18.5 If a solar car is unable to continue because too many energy storage cells have been damaged,

the team may, with the event organiser’s permission, replace cells and continue. The final

ranking of the team will be based on the distance achieved before replacing cells.50

3.19

Route

3.19.1 The event course will be the major highways between Darwin, Northern Territory, and Adelaide,

South Australia—approximately 3020 km.

3.19.2 One printed copy of the official route notes will be provided to each registered team. Additional

copies will be available for download from the Bridgestone World Solar Challenge website from

September 2023.

3.19.3 Mandatory control stops will be established along the route. The locations of the control stops

will be published in the official route notes.

3.19.4 Any detour signs erected by the civil authorities must be observed and the correct route re

joined at the first opportunity.

3.19.5 The event organiser may change the course at any time.

3.20

Timing

3.20.1 The official start time on Day 1 will be 10:00 for all teams.

3.20.2 The official start time each day after Day 1 is 08:00. The official finish time each day is 17:00.

3.20.3 A time penalty of one minute will be imposed for each minute taken past the official finish time,

up to and including 10 minutes past the official finish time. Beyond 10 minutes, each additional

minute will attract a penalty of two minutes. Time penalties incurred in this manner will

determine an adjustment of the official start time the following day.

Example: If a team finishes at 17:07 on Day 2, the official start time for Day 3 will be adjusted to 08:07. If a team

finishes at 17:13 on Day 2, the official start time for Day 3 will be adjusted by 1 × 10 + 2 × 3 minutes to 08:16.

3.20.4 A solar car starting before its official start time will be subject to a penalty of 2 minutes for each

offending minute.

3.21

Tracking

3.21.1 Each solar car must carry a data logging and tracking device provided by the event organiser.

The specifications of the tracking device are described in Regulation 2.9.

3.21.2 Information concerning progress and operation of solar cars may be published during the event.

3.21.3 Satellite tracking data will not be accepted as basis for inter-team protests.

3.22

Solar car drivers and passengers

3.22.1 Only registered solar car drivers may drive the solar car during event hours.

3.22.2 When driving, ballast must be carried in accordance with the provisions of Regulation 2.8.51

3.22.3 Each solar car driver and passenger must have at least two litres of water whenever they start

their stint.

3.22.4 Solar car drivers must change at Adelaide River, about 115km south of Darwin, at a location to

be specified in the Route Notes.

3.22.5 Solar car drivers who will drive the solar car for more than one hour in a day must not drive a

support car within two hours of driving the solar car.

3.23

Driving conduct

3.23.1 Each team must ensure that all vehicles associated with their attendance at the event are driven

in a careful and courteous manner always.

3.23.2 Overtaking must be done in a safe manner and take in to account prevailing conditions.

A team seeking to overtake another team should communicate with the other team via radio, and both teams should

maintain communication throughout the manoeuvre.

3.23.3 The organiser reserves the right to determine if any individual is acting de facto as a part of a

team.

3.23.4 The front and rear escort vehicles must have their amber flashing lights operating whenever

they are escorting the solar car. Support vehicles must not have amber flashing lights operating

if they are not (directly) escorting the solar car.

3.23.5 Except when pushing a disabled solar car to a place of safety, solar cars must not be pushed

along the road. Push starting the solar car is not allowed.

3.23.6 Whilst travelling on the open road, the front escort vehicle must be within 500 m of the solar

car. The solar car must be allowed a safe stopping distance appropriate to the speed and

conditions prevailing.

3.23.7 Whilst travelling on the open road, the rear escort vehicle must be within 5 seconds of the solar

car and must always maintain a safe stopping distance appropriate to the speed and conditions

prevailing.

3.23.8 Solar cars must not be driven on any public road between sunset and sunrise.

3.23.9 Teams must comply with all requirements imposed by road traffic authorities, police, or event

officials.

3.23.10 Solar cars, escort vehicles and other team vehicles may be subject to scrutineering at any time

during the event to ensure compliance with the regulations.

3.24

Stopping

3.24.1 No team vehicle may stop on the road except in an emergency or when required by traffic

conditions.52

Stopping on the road so that you can lift your car off the road is dangerous and will attract a penalty determined by

the Clerk of the Course.

3.24.2 When stopped adjacent to a road, all wheels of all vehicles must be clear of the road.

3.24.3 Wherever possible, vehicles must be parked, and all team activity must occur, at least 10 m from

the edge of the road.

3.24.4 Where any activity takes place less than 2 m from the road, two team members must be

positioned in a place of safety to warn approaching traffic using yellow flags, and to warn the

team of approaching traffic using a whistle.

3.24.5 If a solar car has suffered damage to any component certified by the team’s certifying engineer,

other than tyres, then the team may repair their solar car but may not resume driving until:

• the team’s certifying engineer has recertified the repaired car

• the team has submitted the new certification to the Clerk of the Course

• the Clerk of the Course has approved the resumption of driving.

3.24.6 If a solar car rolls onto its side or roof then the team must withdraw immediately.

3.24.7 Teams wanting to repair their solar car or escort vehicles may, after obtaining approval from the

Clerk of the Course, mark the road at the location where they stopped and trailer their solar car

to a suitable place for making repairs, then return to the location where they stopped and

continue driving.

3.25

Obstructing other road users

3.25.1 Vehicles associated with a team must not obstruct other road users, including other teams.

Infringement of this regulation will incur a penalty for each incident.

3.25.2 To avoid obstructing other road users, each driver of a team vehicle other than the solar car and

front and rear escort vehicles must leave at least 200 metres of space in front of their vehicle so

that other road users can overtake and occupy the space in front of them. If it is safe to do so

and will help clear traffic building up behind a vehicle, drivers of team support vehicles should

pull off the road to allow other road users to pass.

3.26

Control stops

3.26.1 Solar cars must stop for 30 minutes at designated control stops along the route. The locations

of the control stops will be specified in the official route notes provided by the event organiser.

3.26.2 All vehicles must obey control stop speed limits.

3.26.3 Teams must obey all directions given by the control stop manager.53

2.26.4 Teams must park in the spaces designated by control stop officials. Teams may not modify the

parking space in any way. Ground sheets may not be used. No team members other than arriving

occupants may touch the solar car once it is in the parking space.

2.64.4 Occupants must get out of the solar car without assistance from other team members.

Occupants must remove their ballast from their solar car.

3.26.4 Once all occupants are out of the car, the arriving driver, alone, may reconfigure the car for

charging before control stop timing commences.

3.26.5 Control stop timing commences when the arriving driver activates the control stop timing

system.

3.26.6 No team member may touch the solar car for 25 minutes. The solar car may reconfigure itself.

3.26.7 After 25 minutes, the departing driver, alone, may reconfigure the car for driving. The driver and

any passengers may get into the car and prepare for departure. No other team member may

touch the car.

3.26.8 The solar car may depart once 30 minutes have elapsed. The solar car must not depart until

ballast and drinking water are in place, seat belts are fastened, helmets are secured, and the

solar car is ready for the road.

The control stop manager may reset the remaining time to 30 minutes if team members interfere with the solar car

during the 30-minute control stop period.

3.27

Overnight stops

3.27.1 An overnight stop commences when the team stops driving for the day and finishes when the

team starts driving the next day.

3.27.2 The selection (and any cost) of overnight stop locations is the responsibility of each team.

3.27.3 At overnight stops, the solar car and all other vehicles must be parked in such a manner that

they, and all team activities, are conducted at least 10 metres from the edge of any road.

3.27.4 If a team's overnight camp is not immediately adjacent to the road, a marker cone must be

placed on the roadside adjacent to the point the team exited the official route.

3.27.5 The observer, with the Team Manager, must place a mark on the road to indicate the position

at which the solar car stopped. The mark will be the starting line for the following morning.

3.27.6 The solar car must be driven, pushed by hand or carried by hand between the stopping location

and its solar charging location.

3.27.7 There are no restrictions on the number of team members used to reconfigure the solar

collector for charging at overnight stops.54

3.27.8 The energy storage packs of Challenger Class solar cars will be disconnected and sealed

15 minutes after sunset each day. The energy storage packs of Cruiser Class solar cars will be

disconnected and sealed:

• 15 minutes after sunset if the car is not at the end of a stage, or

• 15 minutes after the end of external charging at stage ends.

3.27.9 Energy storage packs will be unsealed 15 minutes before sunrise.

3.29

Trailering

The term ‘trailering’ applies to any means of transport used to carry the solar car.

3.29.1 Teams unable to maintain a minimum speed of 60 km/h on the open road or unable to drive to

the next control stop before it closes must stop and transport their solar car forward to the next

open control stop to get further instructions from the Clerk of the Course.

3.29.2 The solar car must be in safe state whenever it is being transported.

3.30

Withdrawal

3.30.1 A team may withdraw by forwarding a completed withdrawal form (available from the observer)

to the Clerk of the Course. The solar car may be transported to Adelaide to take part in the

displays and the awards ceremony.

3.31

Penalties

3.31.1 The Clerk of the Course may impose demerit points on any team that breaches a regulation.

Demerit points will be published.

3.31.2 Each demerit point given to a Challenger Class team will also incur a time penalty of 30 minutes,

to be served at a control stop nominated by the Clerk of the Course. All Challenger Class time

penalties must be served before arriving at the finish line.

3.31.3 Each demerit point given to a Cruiser Class team will reduce the team’s final score by 1%.

3.31.4 A team accruing three or more demerit points will be excluded from the remainder of the event.

3.31.5 The Clerk of the Course may impose operational restrictions (such as speed restrictions) on any

team.

3.31.6 At any time during the event, the Clerk of the Course may issue a penalty of one demerit point

in response to any of the following behaviours noted by an observer or reported by an official:

• obstructing other road users

• stopping on the road (other than when required by traffic conditions)55

• poor safety protocols (e.g., safety flags, whistles etc.)

• overnight activities too close to the road

• driving without both front and rear escort vehicles

• escort vehicles driving without visible flashing lights

• driving without effective rear vision.

3.31.7 The Clerk of the Course will determine penalties for offences including:

• slipstreaming or pressure wave pushing

• failure to follow the route instructions

• failure to observe a request by police or event officials

• wilful damage or interference to property

• failure to stop at a designated control stop

• exceeding any posted speed limit

• driving at less than 50 km/h in areas with a speed limit of 100 km/h or greater, when not

constrained by traffic

• driving without adequate visibility, due to smoke, dust or rain.

3.31.8 The Clerk of the Course may exclude any team from the event for wilful disregard of any

regulation or of the spirit of the event. These offences include but are not limited to:

• misrepresentation

• wilful obstruction, or aggressive or unsafe driving

• replacement of energy storage cells

• charging of the energy storage system from any unapproved source

• driving the solar car without a rear escort vehicle

• accumulation of three or more demerit points.

3.32

Protests and appeals

3.32.1 A Team Manager may appeal any decision of the Chief Scrutineer to the Jury by lodging a written

Notice of Appeal with the Clerk of the Course within one hour of the notification of the decision.

The Notice of Appeal must detail the grounds for the appeal. An appeal fee of AU$500 will apply.

3.32.2 A Team Manager may lodge a written protest with the Clerk of the Course. Protests must be

lodged before 18:00 on the day of the incident giving rise thereto. Protests will not be accepted

after 18:00 ACST on the final Saturday of the event. The protest shall specify the details of the

incident and a protest fee of AU$200 will apply.56

3.32.3 A Team Manager may appeal any decision of the Clerk of the Course to the Jury by lodging a

written Notice of Appeal with the Clerk of the Course within one hour of the notification of the

decision. The Notice of Appeal must detail the grounds for the appeal. An appeal fee of AU$500

will apply.

3.32.4 Protest/appeal fees will be returned if the protest/appeal is upheld.

3.32.5 The Chief Steward may convene a tribunal to consider any protest or appeal requiring final

resolution. The tribunal may take advice from any party about the incident giving rise to the

decision being appealed. The decision of the tribunal is final and binding.

3.32.6 The decision of the tribunal must be conveyed to the Team Manager lodging the appeal within

one hour of reaching the decision and confirmed in writing within 12 hours to all parties to the

appeal including the Clerk of Course.57

4

Achievement and Celebration

4.1

Event results

4.1.1

Results published throughout the duration of the event are provisional. Results will not be final

until after the determination of any outstanding protests and appeals.

4.2

Completing the course

4.2.1

To complete the course, the solar car must be driven the entire designated route in accordance

with the regulations.

4.2.2

A marshalling point will be established a few kilometres before the finish line. The finish time

for the journey will be determined at the marshalling point.

4.2.3

The solar car must drive to the finish line, escorted by vehicles provided by the organiser. Other

team vehicles will be directed to a long-term parking location.

4.3

Challenger Class

4.3.1

The goal for the Challenger Class is to be the first to arrive at the Adelaide marshalling point, in

accordance with the regulations.

4.3.2

An urban area reference point will be established at the start of the Adelaide urban area. The

location will be published in the official route notes. If team A can keep up with traffic and is

overtaken by team B after the urban area reference point, the arrival time of team B at the

marshalling point will be adjusted to be one minute behind the arrival time of team A. If team A

is overtaken by team B and team A is unable to keep up with traffic at any point after the urban

area reference point, the arrival time of team B will not be adjusted.

4.3.3

Any unserved time penalties must be served at the marshalling point.

4.3.4

The Challenger Cup will be awarded to the Challenger Class team that completes the route with

the earliest finish time (arrival time at the Adelaide marshalling point plus any unserved penalty

time).

4.3.5

Challenger Class teams that complete the route will be ranked by finish time (arrival time at the

Adelaide marshalling point plus any unserved penalty time).

4.3.6

Challenger Class teams that do not complete the route will be ranked behind Challenger Class

teams that complete the route. They will be ranked by the number of control stops completed

before trailering, and within these groups by arrival time at the last control stop completed

before trailering plus unserved time penalties accrued at any time during the event.

4.4

Cruiser Class

4.4.1

The goal for Cruiser Class teams is to design and build a practical solar car and transport people

from Darwin to Adelaide according to a given schedule. Cruiser Class teams will be scored on

energy efficiency and practicality.

4.4.2

Cruiser Classteams must arrive at key staging locations prior to the following target arrival times:

Staging location

Target arrival time

Latest arrival time

Tennant Creek

Saturday, 15:30

Saturday, 17:00

Coober Pedy

Monday, 16:30

Monday, 17:00

Adelaide

Wednesday, 11:30

Wednesday, 14:00

The average on-road speed required to meet these target times is about 75 km/h.

4.4.3

Teams arriving at a staging location after the specified latest arrival time will be deemed to have

not completed the stage.

4.4.4

Teams arriving at a staging location after the specified target arrival time but before the latest

arrival time will have their score reduced by 1 percent for each minute late.

Example: If a team is 7 minutes late at Tennant Creek and 9 minutes late at Adelaide then their score will be multiplied

by 0.99^ (7 + 9) = 0.851.

4.4.5

Cruiser Class teams will spend Saturday night at Tennant Creek and Monday night at Coober

Pedy. Solar car drivers must clock in, but a 30-minute control stop is not required for Cruiser

Class teams at these locations.

4.4.6

Cruiser Class solar cars may be recharged from Electric Vehicle Supply Equipment (EVSE)

provided by the organiser at Tennant Creek and Coober Pedy, between sunset and 23:00. No

other charging from external sources is allowed. The external energy used to recharge the solar

car will be measured by the organisers. The time each Cruiser Class teams spends charging from

external sources will be published, along with the energy used, and will be considered by the

practicality Judges.

4.4.7

Each Cruiser Class solar car completing at least the first stage will be given a final score calculated

by

S = D / E × P × 0.99(l + d)

where

• D is the person-km distance travelled

• E is the nominal external energy use of the solar car, in kilowatt-hours

• P is the practicality score of the solar car, between 0 and 1

• l is the total lateness at staging locations, in minutes

• d is the number of demerit points received by the team.

4.4.8

The Cruiser Cup will be awarded to the Cruiser Class team that completes all stages with the

highest score.

4.4.9

Cruiser Class teams will be ranked in the following groups:

• teams that complete the entire route to Adelaide, ranked by score

• remaining teams that complete the entire route to Coober Pedy, ranked by score

• remaining teams that complete the entire route to Tennant Creek, ranked by score

• all remaining teams, unranked.

4.4.10 Cruiser Class teams arriving at the Adelaide marshalling point before 10:00 on day 6 will not

have their arrival time recognised. No Cruiser Class team will be allowed to proceed to the finish

line until after 10:00 on day 6.

4.4.11 The progressive score of a Cruiser Class team at each control stop will be calculated by

S = D / E × 0.99(l + d)

where

• D is the person-km distance achieved between Darwin and the control stop

• E is the nominal external energy use of the solar car, in kilowatt-hours, prior to arriving at

the control stop

• l is the total lateness at staging locations, in minutes

• d is the number of demerit points received by the team.

4.4.12 Nominal external energy use of a Cruiser Class solar car will be calculated as E0 + E1 + E2, where

E0 is the nominal energy capacity of the energy storage system, E1 is the recharge energy

measured at Tennant Creek and E2 is the recharge energy measured at Coober Pedy. The

nominal energy capacity of a rechargeable electrochemical battery is the sum of the nominal

cell masses in kilograms multiplied by:

• 330 Wh kg-1 for Li-S cells

• 250 Wh kg-1 for Li-ion cells

• 250 Wh kg-1 for Li-polymer cells

• 140 Wh kg-1 for LiFePO4 cells.

4.4.13 The nominal energy capacity of other types of energy storage system will be determined by the

Chief Energy Scientist.

4.4.14 Person-km distance for a leg between consecutive control stops is the minimum number of

occupants in the car during the leg, multiplied by the leg distance.

4.4.15 The practicality score for a Cruiser Class team will be determined by a panel of Judges appointed

by the organiser. Each Judge will allocate a single score, between 0 and 100%, to each team. The

final practicality score for a team will be the arithmetic mean of the individual Judge’s scores.

Judges will be asked to consider:

• design innovation

• environmental impact

• ease of access and egress

• occupant space and comfort

• ease of operation (driving and charging)

• versatility

• style and desirability

• suitability for the declared purpose.

Judges will not be told how much weight to place on each of these criteria.

The number of seats will not contribute to the practicality score, because the benefits of more seats should be reflected

in the person-km distance score.

4.4.16 Cruiser Class teams must prepare a 3-minute video and a brochure that describes their car, how

it will be used, and how it addresses sustainability, mobility and energy resilience. The video and

the car will be presented to the Judges in Adelaide. Teams must allow Judges to get into their

car to test the space and comfort.

4.5

Adventure Class

4.5.1

Adventure Class is non-competitive.

4.5.2

The progress of Adventure class solar cars will be controlled by the Clerk of the Course.

Adventure Class solar cars will be allowed to drive to the finish line only after the first four Challenger Class solar cars

have done so.

4.5.3

Teams in Adventure Class will not be ranked by performance.

4.6

Awards

4.6.1

An awards ceremony will be held in Adelaide on the evening of Friday 27 October 2023 (exact

date, time, and location to be confirmed).

4.6.2

Entry to the awards ceremony is by ticket only.

Tickets for registered team members are included in the team entry fee and pro-rata team member registration fee

outlined in Regulation 1.14. Additional tickets (subject to the capacity of the venue) for friends, family, sponsors, and

public may be purchased. Further details will be provided in the Team Manager’s Guide or via a Team Notice.

4.6.3

The winner of the Challenger Class will receive the Challenger Cup; the winner of the Cruiser

Class will receive the Cruiser Cup.

4.6.4

Additional awards may be presented.

4.6.5

The event organiser reserves the right not to present an award in any given category.

4.6.6

Each team will receive a participation award.

4.6.7

All winning teams must make themselves available for an official media call if required.

4.7

Pack-down facilities

4.7.1

Open-air facilities will be made available for repacking solar cars for shipment. All vehicles,

containers and equipment must be removed by close of business Tuesday 31 October 2023. Any

remaining items will be removed for storage at the entrant’s expense.